Object Detection Using OpenCV and Python

The object detection works on the Viola-Jones algorithm, which was proposed by Paul Viola and Michael Jones. The aforementioned algorithm is based on machine learning. The first step involves training a cascade function with a large amount of negative and positive labeled images. Once the classifier is trained, identifying features, namely “HAAR Features,” are extracted from these training images. HAAR features are essentially rectangular features with regions of bright and dark pixels.

Introduction of Modules used –

OpenCV :-

OpenCV is an open-source library dedicated to solving computer vision problems. Here CV stands for “Computer Vision”.

We can install this modules by using “pip3 install opencv-python” this command.

Open CV helps us in accessing our webcam’s input in real time.

We can also apply Objetc Detection on a pre recorded video or any locally stored imges .

NumPy :-

NumPy is a Python library used for working with arrays. It also has functions for working in domain of linear algebra, fourier transform, and matrices. NumPy was created in 2005 by Travis Oliphant. It is an open source project and you can use it freely.

Time Module :-

We are using this module for calculating the realtime running FPS.

This is totally optional .

How does Object Detection work?

Here we are yolo v3 algorithm for our work . We have pretrained model’s weights and config files with us .

As mentioned already, YOLO which stands for “You only look once” is a single shot detection algorithm which was introduced by Joseph Redmon in May 2016. Although the name of the algorithm may sound strange, it gives a perfect description of this algorithm as it predicts classes and bounding boxes for the whole image in one run of the algorithm.

YOLO performed surprisingly well as compared to the other single-shot detectors of that time in terms of speed and accuracy. It is not the most accurate algorithms when it comes to object detection but certainly, it makes that up with its impressive speed and thus is a good balance between speed and accuracy.

Line By Line Code Explaination

1 – First of all we are importing our modules by using ‘import      keyword’ of python.

2 – In line no. 6 of our code file we are loading of predefined weights and cofig file of our yolo v3 model . This model is capable of providing realtime object detection at 320 fps.

3 – For names we are using coco name file . It has thousand of object’s names in that and it will help us in providing the output.

Just like in this image we have names corresponding to the objects .

4 – At line 11 “cap = cv2.VideoCapture(0)” this Video Capture function is used with value 0 which means our program is going to use machine’s webcam for the main purpose .

5- We can change the value written this class function. We can provide images, videos , URL’s etc.

6- Line 13 and 14 is used for fps calculation . We are initializing two variables to value 0 .

7 – Now we have an infinite loop which is while loop . We are using this because we want to run our process for unlimited amount of time until user closes it.

8 – Geneally CV operations use infinite loops for their function . Take an example of Tesla Car it has its own object detection which starts with the car and closes with the car which is actually controlled by the driver itself . This filed of CV is actually utilizing the power of infinite loops .

9 - The next step will be to run the classifier to detect the face in the video feed and the webcam. The basic steps are; first, we capture the frame from the video feed. Next, the captured frame is converted to grayscale. Finally, the grayscale image is passed through the classifier to detect the required object.

10 - It should be clear that the Viola-Jones algorithm is not restricted to face detection only. Multiple types and numbers of objects in a single frame can be detected using this algorithm. All you need to do is to add multiple layers of cascade classifiers in the program, as per your requirement. Below is the complete code. I have added an additional example to include an additional layer of the classifier on the same image as a comment. You can experiment around with different classifiers using the same basic template.

Future Scope of Object Detection

Object detection is a key ability for most computer and robot vision system. Although great progress has been observed in the last years, and some existing techniques are now part of many consumer electronics (e.g., face detection for auto-focus in smartphones) or have been integrated in assistant driving technologies, we are still far from achieving human-level performance, in particular in terms of open-world learning. It should be noted that object detection has not been used much in many areas where it could be of great help. As mobile robots, and in general autonomous machines, are starting to be more widely deployed (e.g., quad-copters, drones and soon service robots), the need of object detection systems is gaining more importance. Finally, we need to consider that we will need object detection systems for nano-robots or for robots that will explore areas that have not been seen by humans, such as depth parts of the sea or other planets, and the detection systems will have to learn to new object classes as they are encountered. In such cases, a real-time open-world learning ability will be critical.